# Glue Guru

# SAFETY DATA SHEET

Section 1.		Identification of the material and the supplier

Product: 	HSE-Tac Canister

Size:	13.6kg

Product Use: 	Industrial Adhesives

Restriction of Use:	Refer to Section 15

New Zealand Supplier:	Glue Guru

Address:		1016E Great South Road

Penrose, Auckland, 1061

Telephone:	  09 444 4878

Fax Number:	09 442 5975

NZ Emergency No:	0800 764 766(National Poison Centre)

Australian Supplier: 	Glue Guru International Ltd

Address:	1/21 Leakes Road

Laverton North

Melbourne, VIC

Australia

Telephone No:	1300 901 687

Australian Emergency No	13 11 26  (National Poison Centre)

Date of SDS Preparation:	17 June 2025

Section 2.		Hazards Identification

Australia:

Classified as Hazardous according to the Globally Harmonised System of Classification and labelling of Chemicals (GHS) including Work, Health and Safety regulations, Australia

New Zealand:

This substance is hazardous according to the EPA Hazardous Substances (Classification)

Notice 2020

EPA Approval Code:    Gases Under Pressure Mixtures (Flammable) – HSR0002532

###### Pictograms:

Signal Word:  DANGER

| GHS Category | Hazard Code | Hazard Statement |
| --- | --- | --- |
| Flammable gas Cat. 1A | H220 | Extremely flammable gas. |
| Liquified Gas | H280 | Contains gas under pressure may explode if heated. |
| Eye irritation Cat. 2 | H319 | Causes serious eye irritation. |
| Specific target organ toxicity – repeated exposure Cat. 1 | H372 | Causes damage to organs through prolonged or repeated exposure. |
| Hazardous to the aquatic environment chronic Cat. 3 | H412 | Harmful to aquatic life with long lasting effects. |

Prevention Code     Prevention Statement

| P103 | Read carefully and follow all instructions. |
| --- | --- |
| P202 | Do not handle until all safety precautions have been read and understood. |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P260 | Do not breathe dust, fumes, gas, mist, vapours or spray. |
| P264 | Wash hands thoroughly after handling. |
| P270 | Do not eat, drink or smoke when using this product. |
| P273 | Avoid release to the environment. |
| P280 | Wear protective clothing as detailed in SDS Section 8. |

Response Code	Response Statement

| P314 | Get medical advice/attention if you feel unwell. |
| --- | --- |
| P377 | Leaking gas fire: Do not extinguish, unless leak can be stopped safely. |
| P381 | In case of leakage, eliminate all ignition sources. |
| P305 + P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention. |

Storage Code	Storage Statement

| P403 | Store in a well-ventilated place. |
| --- | --- |
| P410 + P403 | Protect from sunlight. Store in a well-ventilated place. |

Disposal Code	 Disposal Statement

| P501 | Dispose of according to the local authorities |
| --- | --- |

#### Section 3.		Composition of hazardous Ingredients

| Ingredients | Wt% | CAS NUMBER. |
| --- | --- | --- |
| n-hexane | 15-20 | 110-54-3 |
| Dimethyl Ether | <10 | 115-10-6 |

Section 4.		First Aid Measures

Routes of Exposure:

If in Eyes	Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention.

If on Skin	Wash skin with soap and water, launder soaked clothing before re-use.  Seek medical attention if needed.

If Swallowed	Do NOT induce vomiting. If vomiting occurs turn patient on side. IF exposed or concerned: Call a POISON CENTER/doctor.

If Inhaled	Remove persons affected by vapour to fresh air. Apply artificial

respiration if patient is not breathing. If experiencing respiratory symptoms: Call a POISON CENTER/doctor.

Most important symptoms and effects, both acute and delayed:

Symptoms:

Ingestion:	Not applicable.

Inhalation:	Not applicable.

Skin:	Not applicable.

Eye:	Causes serious eye irritation.

Chronic:	May cause damage to organs through prolonged or repeated exposure

Section 5.		Fire Fighting Measures

| Hazard Type | Extremely flammable gas canister. |
| --- | --- |
| Hazards from products | In case of fire, stop leak if safe to do so. Vapours are heavier than air and may travel considerable distances to a source of ignition and flashback. May decompose in a fire, giving off toxic and irritant vapours. Hazardous decomposition product(s): Carbon dioxide, Carbon monoxide |
| Suitable Extinguishing media | Water spray, foam, dry powder or CO2 |
| Precautions for firefighters and special protective clothing | Fire fighters should wear complete protective clothing including self-contained breathing apparatus. Do not breathe fumes. If possible remove containers before using water. Keep containers cool by spraying with water if exposed to fire. |
| HAZCHEM CODE | 2YE |

Section 6.		Accidental Release Measures

Use personal protective equipment as detailed in Section 8. Avoid breathing vapours or gas. Ensure adequate ventilation. Remove all ignition sources. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Evacuate non-essential personnel.

Do not allow to enter drains, sewers or watercourses.

Contain and collect spillage with non-combustible, absorbent material e.g. sand, earth, vermiculite or diatomaceous earth. Place into a container for disposal.  Dispose as per Section 13.

Section 7.		Handling and Storage

Handling:

- Read carefully and follow all instructions.

- Do not handle until all safety precautions have been read and understood.

- Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.

- Do not breathe fumes, gas, mist, vapours or spray.

- Wash hands thoroughly after handling.

- Do not eat, drink or smoke when using this product.

- Avoid release to the environment.

- Avoid contact with skin and eyes.

- Flash back is possible over considerable distance.

- Container explosion may occur under fire conditions.

- Eating, drinking and smoking should be prohibited in areas where this material is handled, stored and processed.

- Ensure operatives are trained to minimise exposures.

- Ensure adequate ventilation.

- Keep away from: Elevated temperature.

- Contaminated clothing should be thoroughly cleaned.

- Keep from direct sunlight.

Storage:

- Store locked up.

- Keep container tightly closed.

- Protect from sunlight.

- Containers which are opened must be carefully resealed and kept upright to prevent leakage.

- Keep only in original container.

- Store in a cool/low-temperature, well-ventilated (dry) place away from heat and ignition sources.

- Opened containers should be carefully resealed and stored in an upright position.

- Store at room temperature.

- Store away from acids, bases and strong oxidising agents.

Section 8		Exposure Controls / Personal Protection

WORKPLACE EXPOSURE STANDARDS (provided for guidance only)

TWA	STEL

Substance		ppm   mg/m3	ppm   mg/m3

Hexane (n-Hexane) 		[110-54-3] 			20 	  72		  -	   -

Dimethylether 		[115-10-6] 			400 	766 		500 	958

Workplace Exposure Standard – Time Weighted Average (WES-TWA). The time-weighted average exposure standard designed to protect the worker from the effects of long-term exposure.  Workplace Exposure Standard – Short-Term Exposure Limit (WESSTEL). The 15-minute average exposure standard. Applies to any 15- Minute period in the working day and is designed to protect the worker against adverse effects of irritation, chronic or irreversible tissue change, or narcosis that may increase the likelihood of accidents. The WES-STEL is not an alternative to the WES-TWA; both the short-term and time-weighted average exposures apply.  Workplace Exposure Standards and Biological Exposure Indices FEB 2025 15TH EDITION.

Engineering Controls

Ensure adequate ventilation. Store in a cool/low-temperature, well-ventilated (dry) place away from heat and ignition sources.

Personal Protection Equipment

| Eyes | If there is a risk to eyes use safety glasses & goggles to approved standard such as EN166(EU) |
| --- | --- |
| Hands | Wear impervious gloves (EN374). |
| Skin | Use impervious overalls. |
| Respiratory | In case of inadequate ventilation wear respiratory protection. A suitable mask with filter type AX may be appropriate. |
| General | Keep good industrial hygiene. Wear appropriate personal protective equipment, avoid direct contact. Avoid breathing gas. IF exposed: Wash immediately with water. Wash contaminated clothing before reuse. Do not eat, drink or smoke at the work place. |

Section 9		Physical and Chemical Properties

| Appearance | Gas Canister with liquid |
| --- | --- |
| Colour | Clear/Blue |
| Odour | Solvent odour |
| Odour Threshold | Not available |
| pH | 7 @ 5g/l @ 20 °C |
| Boiling Point | 69oC |
| Melting Point | -95 °C |
| Freezing Point | Not applicable |
| Flash Point | -25.99 °C |
| Flammability | Highly Flammable Gas |
| Upper and Lower Explosive Limits | 1.2 – 7.7% |
| Vapour Pressure | Not available |
| Vapour Density | Not available |
| Relative Density | 0.65 g/mL at 25 °C |
| Solubilities | insoluble |
| Partition Coefficient | Not available |
| Auto-ignition Temperature | 234.00C |
| Decomposition Temperature | Not applicable |
| Kinematic Viscosity | Not applicable |
| Particle Characteristics | Not applicable |

Section 10.	Stability and Reactivity

| Stability of Substance | This product is stable under normal conditions. |
| --- | --- |
| Possibility of hazardous reactions | Under normal conditions of storage and use, hazardous reactions will not occur. |
| Conditions to Avoid | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Keep from direct sunlight. Keep away from: Elevated temperature. Do not spray on an open flame or other ignition source. |
| Incompatible Materials | Acids, Bases, Strong oxidising agents. |
| Hazardous Decomposition Products | Carbon dioxide, Carbon monoxide |

Section 11		Toxicological Information

Acute Effects:

| Oral | Not applicable. Acute Toxicity Estimate Mixture Calculation: LD50 >2000 mg/kg bw/day |
| --- | --- |
| Dermal | Not applicable. Acute Toxicity Estimate Mixture Calculation: LD50 >2000 mg/kg bw/day |
| Inhalation | Not applicable. Acute Toxicity Estimate Mixture Calculation: LC50 >20000 ppmV |
| Eye | Causes severe irritation to eyes. |
| Skin | Not applicable. |

Chronic Effects:

| Carcinogenicity | Not applicable. |
| --- | --- |
| Reproductive Toxicity | Not applicable. |
| Germ Cell Mutagenicity | Not applicable. |
| Aspiration | Not applicable. |
| STOT/SE | Not applicable. |
| STOT/RE | Causes damage to organs through prolonged or repeated exposure. |

Section 12.	Ecotoxicological Information

Harmful to aquatic life with long lasting effects.

| Persistence and degradability | No data available |
| --- | --- |
| Bioaccumulation | No data available |
| Mobility in Soil | No data available |
| Other adverse effects | No data available |

Do not allow to enter waterways.

Section 13.	Disposal Considerations

Dispose of this material and its container to hazardous or special waste collection point.

Precautions or conditions to avoid. Do not pierce or burn container, even after use. Make

sure that packaging is completely empty before recycling. Containers of this material may be

hazardous when empty since they retain product residue.

Section 14		Transport Information

This product is classified as Dangerous Goods according to the Australian Code for the Transport of Dangerous Goods by Road and Rail (ADG Code) (7th edition).

This product is classified as a Dangerous Good for transport in NZ; NZS 5433:2020

Road, Rail, Sea and Air Transport

| UN No | 3161 |
| --- | --- |
| Class - Primary | 2 |
| Packing Group | Not assigned |
| Proper Shipping Name | LIQUIFIED GAS – FLAMMABLE |
| Marine Pollutant | No |

Section 15		Regulatory Information

Australia:

Classified as Hazardous according to the Globally Harmonised System of Classification and labelling of Chemicals (GHS) including Work, Health and Safety regulations, Australia

Not classified as a Scheduled Poison according to the Standard for the Uniform Scheduling of Medicines and Poisons (SUSMP).

New Zealand:

This substance is hazardous according to the EPA Hazardous Substances (Classification) Notice 2020

EPA Approval Code:  Gases Under Pressure Mixtures (Flammable) – HSR0002532

Trigger Quantities for this substance:

| Certified Handler | Not required |
| --- | --- |
| Location Certificate | 100kg |
| Tracking Trigger Quantities | Not required |
| Signage Trigger Quantities | 250kg |
| Emergency Response Plan trigger Quantities | 300kg |
| Secondary Containment trigger Quantities | 300kg |
| Fire Extinguisher | 50kg = 1 off |
| Restrictions of use | None |

Section 16		Other Information

Glossary

Cat	Category

EC50	Median effective concentration.

EEL	Environmental Exposure Limit.

EPA	Environmental Protection Authority

HSNO	Hazardous Substances and New Organisms.

HSW	Health and Safety at Work.

LC50	Lethal concentration that will kill 50% of the test organisms 	inhaling or ingesting it.

LD50	Lethal dose to kill 50% of test animals/organisms.

LEL	Lower explosive level.

OSHA	American Occupational Safety and Health Administration.

TEL	Tolerable Exposure Limit.

TLV	Threshold Limit Value-an exposure limit set by responsible 	authority.

UEL	Upper Explosive Level

WES	Workplace Exposure Limit

References:

Australia:

- Preparation of Safety Data Sheets for Hazardous Chemicals Code of Practice.

- Standard for the Uniform Scheduling of Medicines and Poisons.

- Australian Code for the Transport of Dangerous Goods by Road & Rail.

- Model Work Health and Safety Regulations, Schedule 10: Prohibited carcinogens,

restricted carcinogens and restricted hazardous chemicals.

- Workplace exposure standards for airborne contaminants, Safe work Australia.

- American Conference of Industrial Hygienists (ACGIH).

- Globally Harmonised System of classification and labelling of chemicals.

New Zealand:

- EPA Hazardous Substances (Safety Data Sheets) Notice 2017

- Workplace Exposure Standards and Biological Exposure Indices FEB 2025 15th edition.

- Assigning a hazardous substance to a HSNO Approval (Aug 2013).

- Transport of Dangerous goods on land NZS 5433:2020

- HSW (Hazardous Substances) Regulations 2017

Disclaimer

This document has been issued by the Glue Guru and serves as their Safety Data Sheet ('SDS'). It is based on information concerning the product which has been provided to the Glue Guru or obtained from third party sources and is believed to represent the current state of knowledge as to the appropriate safety and handling precautions for the product at the time of issue. Further clarification regarding any aspect of the product should be obtained directly from the manufacturer. While Glue Guru have taken all due care to include accurate and up-to-date information in this SDS, it does not provide any warranty as to accuracy or completeness. As far as lawfully possible, Glue Guru accept no liability for any loss, injury or damage (including consequential loss) which may be suffered or incurred by any person as a consequence of their reliance on the information contained in this SDS

The information herein is given in good faith, but no warranty, express or implied is made.

Please contact the New Zealand distributor, Glue Guru, if further information is required.

Issue Date:		17 June 2025                        Review Date:		17 June 2030