# Glue Guru

# SAFETY DATA SHEET

Section 1.		Identification of the material and the supplier

Product: 	Roof-Tac Aerosol

Size:

Product Use: 	Industrial Adhesive

New Zealand Supplier:	Glue Guru

Address:		1016E Great South Rd

Penrose, Auckland

Telephone:	64 9 444 4878

Fax Number:	64 9 442 5975

NZ Emergency No:	0800 766 764 (National Poison Centre)

Australian Supplier: 	Glue Guru

Address:	1/21 Leakes Road

Laverton North, VIC

Australia

Telephone No:	1300 901 687

E-mail:	enquiries@glueguru.com.au

Australian Emergency No	13 11 26  (National Poison Centre)

Date of MSDS Preparation:	12 June 2025

Section 2.		Hazards Identification

Australia:

Classified as Hazardous according to the Globally Harmonised System of Classification and labelling of Chemicals (GHS) including Work, Health and Safety regulations, Australia

New Zealand:

This substance is hazardous according to the EPA Hazardous Substances (Classification)

Notice 2020

EPA Approval Code:    Aerosols (Flammable, Carcinogenic) – HSR0002517

###### Pictograms:

Signal Word:  DANGER

| GHS Classification and Category | Hazard Code | Hazard Statement |
| --- | --- | --- |
| Aerosol Cat. 1 | H222 | Extremely flammable aerosol. |
|  | H229 | Pressurized container: May burst if heated. |
| Skin irritation Cat. 2 | H315 | Causes skin irritation. |
| Eye irritation Cat. 2 | H319 | Causes serious eye irritation. |
| Carcinogenicity Cat. 2 | H351 | Suspected of causing cancer. |
| Specific target organ toxicity – repeated exposure Cat. 2 | H373 | May cause damage to organs through prolonged or repeated exposure. |

| Prevention Code | Prevention Statement |
| --- | --- |
| P103 | Read carefully and follow all instructions. |
| P201 | Obtain special instructions before use. |
| P202 | Do not handle until all safety precautions have been read and understood. |
| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| P211 | Do not spray on an open flame or other ignition source. |
| P251 | Do not pierce or burn, even after use. |
| P260 | Do not breathe fumes, gas, mist, vapours or spray. |
| P264 | Wash hands thoroughly after handling. |
| P280 | Wear protective clothing as detailed in SDS Section 8. |

| Response Code | Response Statement |
| --- | --- |
| P314 | Get medical advice/attention if you feel unwell. |
| P302 + P352 | IF ON SKIN: Wash with plenty of water. |
| P305 + P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| P308 + P313 | IF exposed or concerned: Get medical advice/ attention. |
| P332 + P313 | If skin irritation occurs: Get medical advice/ attention. |
| P337 + P313 | If eye irritation persists: Get medical advice/attention. |
| P362+P364 | Take off contaminated clothing and wash before reuse. |

| Storage Code | Storage Statement |
| --- | --- |
| P405 | Store locked up. |
| P410 + P412 | Protect from sunlight. Do not expose to temperatures exceeding 50 °C. |

| Disposal Code | Disposal Statement |
| --- | --- |
| P501 | Dispose of according to Local Regulations or Authorities |

#### Section 3.		Composition of hazardous Ingredients

| Ingredients | Wt% | CAS NUMBER. |
| --- | --- | --- |
| dichloromethane | 20 - 30 | 75-09-2 |
| Propane | 5 - 10 | 74-98-6 |
| Isobutane | 5 - 10 | 75-28-5 |

Section 4.		First Aid Measures

Routes of Exposure:

If in Eyes	Hold eyelids apart and flush eyes with plenty of water for at least 15 minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If irritation develops and persists, get medical attention.

If on Skin	Wash skin with plenty of water. Take off contaminated clothing. If skin irritation or rash occurs: Get medical advice/attention.

If Swallowed	DO NOT induce vomiting. Wash out mouth thoroughly with water. Never give anything to the mouth of an unconscious person.   Seek medical attention if needed.

If Inhaled	Remove person to fresh air. Remove contaminated clothing and loosen             remaining clothing. Allow person to assume most comfortable position                                                    and keep warm. Keep at rest until fully recovered. Immediately call a POISON CENTER or doctor/physician.

Most important symptoms and effects, both acute and delayed

Symptoms:

Swallowed:	Not applicable.

Inhalation:	Not applicable.

Skin:	Causes skin irritation.

Eyes: 	Causes serious eye irritation.

Chronic:	Suspected of causing cancer.  May cause damage to organs through prolonged or repeated exposure.

Notes to Doctor:	Treat symptomatically. Do not give adrenaline or other stimulants.

Section 5.		Fire Fighting Measures

| ###### Hazard Type | Extremely flammable Aerosol. The vapours are denser than air and may travel along the ground. Distance ignition possible. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| --- | --- |
| Hazards from products | Combustion produces irritating gases. Combustion produces dangerous gases. Hydrogen chloride. carbon oxides. Chlorine. Phosgene. Thermal decomposition can lead to the escape of irritating gases and vapours. |
| Suitable Extinguishing media | Dry chemical, CO2, or water spray or regular foam. Making extinguishing agents environment-friendly. Do not use a heavy water stream. |
| Precautions for firefighters and special protective clothing | Do not enter fire area without proper protective equipment, including respiratory protection. Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent firefighting water from entering the environment. |
| HAZCHEM CODE | 2YE |

Section 6.		Accidental Release Measures

Measures for personal safety:

Wear personal protective equipment. Where excessive vapour, mist, or dust may result, use

approved respiratory protection equipment. No flames, no sparks. Eliminate all sources of

ignition. No open flames, no sparks, and no smoking. Do not breathe vapours, mist, spray. Avoid contact with skin and eyes. Evacuate unnecessary personnel. Provide adequate ventilation.

Environmental measures:

Do not allow to enter drains or water courses. Notify authorities if liquid enters sewers or public waters.

Methods and material for containment and cleaning up:

Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Place in a suitable container for disposal in accordance with the waste regulations (see Section 13). Store away from other materials.

Section 7.		Handling and Storage

Precautions for Handling:

- Read carefully and follow all instructions.

- Obtain special instructions before use.

- Do not handle until all safety precautions have been read and understood.

- Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No

smoking.

- Do not spray on an open flame or other ignition source.

- Do not pierce or burn, even after use.

- Do not breathe fumes, gas, mist, vapours or spray.

- Wash hands thoroughly after handling.

- Wear protective clothing as detailed in SDS Section 8.

- Ensure good ventilation of the workstation.

- Do not spray on an open flame or other ignition source.

- Pressurized container. Protect from sunlight and do not expose to temperatures

exceeding 50°C.

- Avoid contact with skin and eyes.

- Do not eat, drink or smoke when using this product.

- Keep away from direct sunlight.

Precautions for Storage:

- Store away from incompatible materials: Strong bases, strong acids.

- Store in a well-ventilated place.

- Store locked up.

- Protect from sunlight. Do not expose to temperatures exceeding 50 °C.

- Keep only in the original container and keep tightly closed.

- Store at ambient temperature.

Section 8		Exposure Controls / Personal Protection

WORKPLACE EXPOSURE STANDARDS (provided for guidance only)

TWA	STEL

Substance		ppm   mg/m3	ppm   mg/m3

Methylene chloride (Dichloromethane) [75-09-2] 		 50 	 174		   -	  -

Workplace Exposure Standard – Time Weighted Average (WES-TWA). The time-weighted average exposure standard designed to protect the worker from the effects of long-term exposure.  Workplace Exposure Standard – Short-Term Exposure Limit (WESSTEL). The 15-minute average exposure standard. Applies to any 15- Minute period in the working day and is designed to protect the worker against adverse effects of irritation, chronic or irreversible tissue change, or narcosis that may increase the likelihood of accidents. The WES-STEL is not an alternative to the WES-TWA; both the short-term and time-weighted average exposures apply.  Workplace Exposure Standards and Biological Exposure Indices FEB 2025 15TH EDITION.

DNEL & PNEC

| isobutane (75-28-5) & Propane (74-98-6) |  |
| --- | --- |
| DNEL/DMEL (additional information) |  |
| Additional information | None established. |
| PNEC (additional information) |  |
| Additional information | None established. |
| dichloromethane; methylene chloride (75-09-2) |  |
| DNEL/DMEL (Workers) |  |
| Long-term - systemic effects, dermal | 12 mg/kg bodyweight/day |
| Long-term - systemic effects, inhalation | 176 mg/m3 |
| DNEL/DMEL (General population) |  |
| Long-term - systemic effects,oral | 0.06 mg/kg bodyweight/day |
| Long-term - systemic effects, inhalation | 44 mg/m3 |
| Long-term - systemic effects, dermal | 5.82 mg/kg bodyweight/day |
| PNEC (Water) |  |
| PNEC aqua (freshwater) | 0.31 mg/l |
| PNEC aqua (marine water) | 0.031 mg/l |
| PNEC aqua (intermittent, freshwater) | 0.27 mg/l |
| PNEC aqua (intermittent, marine water) | 0.027 mg/l |
| PNEC (Sediment) |  |
| PNEC sediment (freshwater) | 2.57 mg/kg dwt |
| PNEC sediment (marine water) | 0.26 mg/kg dwt |
| PNEC (Soil) |  |
| PNEC soil | 0.33 mg/kg dwt |
| PNEC (STP) |  |
| PNEC sewage treatment plant | 26 mg/l |

Engineering Controls

Provide adequate ventilation. No flames, no sparks. Eliminate all sources of ignition. Emergency

eye wash fountains and safety showers should be available in the immediate vicinity of any

potential exposure.

Personal Protection Equipment

| Eyes | Wear protective safety goggles EN166 |
| --- | --- |
| Hands | Wear suitable gloves tested to EN374. Recommendation: Suitable chemical resistant safety gloves (EN 374) also with prolonged, direct contact (Recommended: Protective index 6, corresponding > 480 minutes of permeation time according to EN 374): e.g. nitrile rubber (>=0.4 mm), butyl rubber (>=0.7 mm) and others. |
| Skin | Wear suitable protective clothing. Overall. Standard. EN 13034 |
| Respiratory | Wear respiratory protection when in the presence of vapour, dust, and aerosols. CE-approved respirator for organic vapors and solvents (type AX, brown). |
| General | If on skin, take off contaminated clothing. Keep away from food, drink and animal feedingstuffs. Avoid contact with skin and eyes. Wash hands before breaks and after work. Do not eat, drink or smoke when using this product. |

Section 9		Physical and Chemical Properties

| Appearance | Aerosol |
| --- | --- |
| Colour | Blue |
| Odour | Solvent odour |
| Odour Threshold | Not available |
| pH | Not available |
| Boiling Point | 400C |
| Melting Point | -970C |
| Freezing Point | Not available |
| Flash Point | -90 °C - [Closed cup] |
| Flammability | Not available |
| Upper and Lower Explosive Limits | 12% - 19% |
| Vapour Pressure | 4.83 bar @ 21.1 °C |
| Vapour Density | (Air=1) 2.15 |
| Relative Density | 1.3 g/mL @ 25 °C |
| Water Solubility | Insoluble (Hansen solubility test parameter) |
| Partition Coefficient: | Log Pow: 1.25 |
| Auto-ignition Temperature | > 556.1 °C |
| Decomposition Temperature | Not available |
| Kinematic Viscosity | Not available |
| Particle Characteristics | Not available |

Section 10.	Stability and Reactivity

| Stability of Substance | This product is stable under normal conditions. |
| --- | --- |
| Reactivity | Extremely flammable aerosol. Pressurised container: May burst if heated. The product is non-reactive under normal conditions of use, storage and transport. |
| Possibility of hazardous reactions | Stable under normal conditions. |
| Conditions to Avoid | Avoid contact with hot surfaces. Heat. No flames, no sparks. Eliminate all sources of ignition. |
| Incompatible Materials | Strong acids. Strong oxidizers. Strong alkalis. |
| Hazardous Decomposition Products | Combustion produces dangerous gases. Hydrogen chloride. carbon oxides. Chlorine. Phosgene. |

Section 11		Toxicological Information

Acute Effects:

| Swallowed | Not applicable. |
| --- | --- |
| Dermal | Not applicable. |
| Inhalation | Not applicable. |
| Eye | Causes serious eye irritation. |
| Skin | Causes skin irritation. |

Chronic Effects:

| Carcinogenicity | Suspected of causing cancer. |
| --- | --- |
| Reproductive Toxicity | Not applicable. |
| Germ Cell Mutagenicity | Not applicable. |
| Aspiration | Not applicable. |
| STOT/SE | Not applicable. |
| STOT/RE | May cause damage to organs through repeated or prolonged exposure. |

Individual component information:

| propane (74-98-6) |  |
| --- | --- |
| LC50 Inhalation - Rat [ppm] | 20000 ppm/4h |
| dichloromethane; methylene chloride (75-09-2) |  |
| LD50 oral rat | > 2000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 401 (Acute Oral Toxicity) |
| LD50 dermal rat | > 2000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 402 (Acute Dermal Toxicity) |
| LC50 Inhalation - Rat | 49000 mg/m3 mouse |

Section 12.	Ecotoxicological Information

Not classified as hazardous to the aquatic environment.

Toxicity:

| dichloromethane; methylene chloride (75-09-2) |  |
| --- | --- |
| LC50 - Fish [1] | 193 mg/l Test organisms (species): Pimephales promelas |
| EC50 - Crustacea [1] | 27 – 109 mg/l |

Persistence and degradability:

| Roof-Tac (Sprayable Contact Adhesive) |  |
| --- | --- |
| Persistence and degradability | Not established. |
| propane (74-98-6) |  |
| Persistence and degradability | The substance is readily biodegradable. Unlikely to persist. |
| isobutane (75-28-5) |  |
| Persistence and degradability | The substance is readily biodegradable. Unlikely to persist. |

Bioaccumulative potential:

| Roof-Tac (Sprayable Contact Adhesive) |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Pow) | 1.25 |
| Bioaccumulative potential | Not established. |
| propane (74-98-6) |  |
| Partition coefficient n-octanol/water (Log Pow) | 2.36 |
| Partition coefficient n-octanol/water (Log Kow) | Not applicable for gas mixtures. |
| Bioaccumulative potential | Not expected to bioaccumulate due to the low log Kow (log Kow < 4). See section 9. |
| isobutane (75-28-5) |  |
| Partition coefficient n-octanol/water (Log Pow) | 2.76 |
| Partition coefficient n-octanol/water (Log Kow) | Not applicable for gas mixtures. |
| Bioaccumulative potential | Not expected to bioaccumulate due to the low log Kow (log Kow < 4). See section 9. |
| Dichloromethane; methylene chloride (75-09-2) |  |
| Partition coefficient n-octanol/water (Log Pow) | 1.25 @ 200C |

Mobility in soil:

| propane (74-98-6) |  |
| --- | --- |
| Ecology - soil | Because of its high volatility, the product is unlikely to cause ground or water pollution. Partition into soil is unlikely. |
| isobutane (75-28-5) |  |
| Ecology - soil | Because of its high volatility, the product is unlikely to cause ground or water pollution. Partition into soil is unlikely. |

Avoid release to the environment.

Section 13.	Disposal Considerations

Disposal Method:

Dispose of this material and its container to hazardous or special waste collection point.  Do

Make sure that packaging is completely empty before recycling. Containers of this material may

be hazardous when empty since they retain product residue.

Precautions or methods to avoid: Pressurized container:  May burst if heated. Do not drill or burn even after use.

Section 14		Transport Information

This product is classified as Dangerous Goods according to the Australian Code for the Transport of Dangerous Goods by Road and Rail (ADG Code) (7th edition).

This product is classified as a Dangerous Good for transport in NZ ; NZS 5433:2020 and SNZ HB 5433:2021

Road, Rail, Sea and Air Transport

| UN No | 1950 |
| --- | --- |
| Class - Primary | 2 |
| Packing Group | N/A |
| Proper Shipping Name | AEROSOLS |
| Marine Pollutant | No |
| Special Provisions | 63, 190, 277, 344, 327 |

Section 15		Regulatory Information

Australia:

Classified as Hazardous according to the Globally Harmonised System of Classification and labelling of Chemicals (GHS) including Work, Health and Safety regulations, Australia

Classified as a Schedule 5 Poison according to the Standard for the Uniform Scheduling of Medicines and Poisons (SUSMP).

New Zealand:

This substance is hazardous according to the EPA Hazardous Substances (Classification)

Notice 2020

EPA Approval Code:  Aerosols (Flammable, Carcinogenic) – HSR0002517

| HSW (HS) Regulations 2017 and EPA Notices | Trigger Quantity |
| --- | --- |
| Certified Handler | Not required |
| Location Certificate | 3000L AWC |
| Tracking Trigger Quantities | Not required |
| Signage Trigger Quantities | 3000L AWC |
| Emergency Response Plan | 1000L |
| Secondary Containment | 1000L |
| Fire Extinguisher | 3000L AWC = 1 off |
| Restriction of Use | Only use for the intended purpose. |

Section 16		Other Information

Glossary

Cat	Category

EC50	Median effective concentration.

EEL	Environmental Exposure Limit.

EPA	Environmental Protection Authority

HSNO	Hazardous Substances and New Organisms.

HSW	Health and Safety at Work.

LC50	Lethal concentration that will kill 50% of the test organisms 	inhaling or ingesting it.

LD50	Lethal dose to kill 50% of test animals/organisms.

LEL	Lower explosive level.

OSHA	American Occupational Safety and Health Administration.

TEL	Tolerable Exposure Limit.

TLV	Threshold Limit Value-an exposure limit set by responsible 	authority.

UEL	Upper Explosive Level

WES	Workplace Exposure Limit

References:

Australia:

- Preparation of Safety Data Sheets for Hazardous Chemicals Code of Practice.

- Standard for the Uniform Scheduling of Medicines and Poisons.

- Australian Code for the Transport of Dangerous Goods by Road & Rail.

- Model Work Health and Safety Regulations, Schedule 10: Prohibited carcinogens,

restricted carcinogens and restricted hazardous chemicals.

- Workplace exposure standards for airborne contaminants, Safe work Australia.

- American Conference of Industrial Hygienists (ACGIH).

- Globally Harmonised System of classification and labelling of chemicals.

New Zealand:

- EPA Hazardous Substances (Safety Data Sheets) Notice 2017

- Workplace Exposure Standards and Biological Exposure Indices FEB 2025 15th edition.

- Assigning a hazardous substance to a HSNO Approval (Aug 2013).

- Transport of Dangerous goods on land NZS 5433:2020

- HSW (Hazardous Substances) Regulations 2017

Disclaimer

This document has been issued by the Glue Guru and serves as their Safety Data Sheet ('SDS'). It is based on information concerning the product which has been provided to the Glue Guru or obtained from third party sources and is believed to represent the current state of knowledge as to the appropriate safety and handling precautions for the product at the time of issue. Further clarification regarding any aspect of the product should be obtained directly from the manufacturer. While Glue Guru have taken all due care to include accurate and up-to-date information in this SDS, it does not provide any warranty as to accuracy or completeness. As far as lawfully possible, Glue Guru accept no liability for any loss, injury or damage (including consequential loss) which may be suffered or incurred by any person as a consequence of their reliance on the information contained in this SDS

The information herein is given in good faith, but no warranty, express or implied is made.

Please contact the New Zealand distributor, Glue Guru, if further information is required.

Issue Date:		12 June 2025                        Review Date:		12 June 2030